ORDINANCE No.  177433

Establish Wildfire Hazard Zones to regulate types of roofing materials (Ordinance; amend Title 24, add Code Chapter 24.51)

The City of Portland ordains:

Section 1.  The Council finds:

- Portland’s forested areas represent scenic, environmental and recreational treasures for its citizens.

- Many of these areas have been developed with residential and institutional structures.

- In August 2001 a five-alarm wildfire in the Bluffs area of North Portland threatened hundreds of structures, including the University of Portland, and strained the resources of the Portland Fire & Rescue (PF&R) risking a chance of cataclysmic loss of property and risk to human life.

- The potential exists for similar wildfires that could result in loss of life and homes being destroyed, with negative effects on natural resources from erosion and landslides.

- Fire and building jurisdictions throughout the country have modified building and planning codes to reduce the risk of catastrophic fires in heavily vegetated urban areas.

- In recognition of the unique fire problem posed by wildfires that might occur in areas of housing development, the State of Oregon, in 1996, adopted Chapter 629, Division 44, Oregon Administrative Rule (Criteria For Determination of Wildfire Hazard Zones) which establishes criteria for the identification and classification of Wildfire Hazard Zones.

- Large tracts of land in Portland meet the criteria of “Wildfire Hazard Zone” as established by the administrative rules adopted by the Oregon Department of Forestry (ODF). OAR Chapter 629, Division 044, Subsections 0200 - 0260 (Criteria For Determination of Wildfire Hazard Zones).

- On September 19, 2001, Council approved Resolution No. 36028, which identified a process to adopt an Urban Wildfire Hazard Zone policy.

- The process set forth in Resolution No. 36028 includes the adoption of Wildfire Hazard Zones in certain areas within Portland as the first phase.

- PF&R worked with the ODF and Metro to assign and map hazard values for properties within Portland City limits based on topography, fire weather, vegetative fuel and fuel distribution in accordance with OAR Chapter 629, Division 044, Sections 0200 – 0260  (Criteria For Determination of Wildfire Hazard Zones).

- Areas with a composite hazard rating of 5 or higher as defined in OAR Chapter 629, Division 44 Subsection 0220 were identified as potential Wildfire Hazard Zones.

- The zones were further refined by removing some tracts, such as but not limited to golf courses, schools, cemeteries, industrial land and developed park where fire spread would be unlikely.

- A map attached as Exhibit 1 identifies the proposed boundaries of Wildfire Hazard Zones for the city.

- The State of Oregon allows adoption of Oregon Structural Specialty Code (OSSC) Appendix 5 (Wildfire Hazard Mitigation) by the City of Portland.  Appendix 5 provides minimum standards for buildings located in or adjacent to vegetated areas subject to wildfires.

- The State of Oregon One and Two Family Dwelling Specialty Code (OTFDSC) provides additional construction standards for wildfire hazard mitigation for dwellings and accessory structures located in or adjacent to vegetated areas subject to wildfires when Wildfire Hazard Zones are adopted by a municipality as proposed in this ordinance.

- The OSSC Appendix 5 and Section 326 of the OTFDSC (Wildfire Hazard Mitigation), set forth more stringent roofing requirements for structures located inside Wildfire Hazard Zones.

- Property owners with structures located on properties located in designated Wildfire Hazard Zones will be required to replace the entire roof surface with wildfire compliant roofing when reroofing 50% or more of the roof surface in compliance with OSSC Appendix 5 or OTFDSC Section 326, as applicable.

- The fire risk of any particular land parcel can change as development and environmental events occur. Authorization for an administrative exemptions process, based upon the criteria established in OAR Chapter 629, Division 44, Subsections 0200 through 0260 would be desirable.

NOW, THEREFORE, the Council directs:

a.  Portland City Code 24.10.040A, Structural Specialty Code, is amended as follows:

- Structural Specialty Code.  The provisions of the State of Oregon Structural Specialty Code 1998 Edition, as published by the International Conference of Building Officials known as the Uniform Building Code (UBC), Volumes 1 through 3 1997 Edition and amended by the Building Codes Division of the Oregon Department of Consumer and Business Services, including Appendix Chapter 5, is hereby adopted by reference. The Structural Specialty Code is on file in the Development Services Center of the City of Portland.

The Uniform Building Code Standards, to which reference is made in various parts of this Title, shall be the Uniform Building Code Standards included in the Uniform Building Code referenced above.

b.  City Code Title 24, Building Regulations, is amended by adding a new chapter as follows:

24.51 WILDFIRE HAZARD ZONES

24.51.010 Purpose.

The purpose of this Chapter is to adopt the criteria that will be used to specify areas of the City to be classified as Wildfire Hazard Zones, so that roof materials may be limited.

24.51.020 Definitions

The definitions contained in this Section relate to Wildfire Hazard zones and considerations outlined in this Chapter.

- Chief means the Chief of the City of Portland Bureau of Fire, Rescue and Emergency Services or the Chief’s duly authorized representative.

- Department of Forestry (DOF) means the State of Oregon Department of Forestry.

- Wildfire Hazard Zone means those areas of the City as determined by the Chief that rate a minimum score of 5 or higher using the following criteria developed by DOF:

- Topography hazard factor value

- Natural vegetative fuel hazard factor value

- Natural vegetative fuel distribution hazard factor value

- Wildfire Hazard Zone Map  the WHZM attached to this ordinance and as it may be amended from time to time based on the criteria herein.

- Hazard Factor Hazard Factors are topography, certain natural vegetative fuels and natural, vegetative fuel distribution.  Any of these factors, or a combination thereof, may cause an area of the City to be included within a Wildfire Hazard Zone.

- Topography Hazard Factor Value means the hazard value as determined by DOF associated with site slope which effects the fire spread velocity.

- Natural Vegetative Hazard Factor Value means the numerical value assigned by DOF, extrapolated from the “Aids to Determining Fuel Models for Estimating Fire Behavior” published by the Forest Service, USDA Intermountain Forest and Range Experiment Station in 1982 as General Technical Report INT-122, for various common vegetation.

- Natural Vegetative Fuel Distribution Hazard Factor Value means the numerical value assigned by DOF for the percentage of site that is covered by vegetation described in “G” above.

24.51.030 Wildfire Hazard Zone Map Adoption.

- Wildfire Hazard Zone Map Adoption.

- A Wildfire Hazard Zone Map (WHZM) has been developed for the City of Portland through a review of topography, weather, type vegetation and fuel density.  This map is dated October 11, 2002.

- The WHZM dated October 11, 2002, is hereby adopted by reference and incorporated into this ordinance.

- The Chief shall provide the Director with a copy of the official map adopted in Subsection one of this Section. Copies of the map shall be available for review in the Development Services Center, First Floor 1900 SW 4th Avenue, Portland Oregon.

- Revisions to the Wildfire Hazard Zone Map.

1.	The WHZM may be amended from time to time to either include or exclude properties as the facts may warrant.

2. 	The Chief shall have the authority to revise the Wildfire Hazard Zone Map.

3.	All Wildfire Hazard Zone map revisions shall be determined using the criteria set forth below. Any site having a cumulative hazard value of five (5) or more shall be included in a wildfire hazard zone.

- Topography Hazard Factor Value. The topography hazard value shall be calculated as follows:

1)	Determine site slope using the appropriate 7.5 minute quadrangle map published by the U.S. Geological Survey, USDI.

2)	Select appropriate hazard value using Table 1.

TABLE 1

APPROPRIATE TOPOGRAPHY HAZARD FACTOR VALUE

| Site Slope as determined by the 7.5 minute quadrangle map | Hazard Value |
| --- | --- |
| Slopes 00 to < 03% | 0 |
| Slopes 03 to < 12% | 1 |
| Slopes 12 to < 20% | 2 |
| Slopes 20% or greater | 3 |

- Natural Vegetative Fuel Hazard Factor Value.  The natural vegetative fuel hazard value shall be calculated as follows:

- Divide the jurisdiction into geographic areas which best describe the natural vegetation expected to occupy sites for the next 10 to 15 years.

- Select the appropriate hazard value from Table 2.

TABLE 2

NATURAL VEGETATIVE FUEL HAZARD FACTOR VALUE

| Natural Vegetative Fuel Description 1 |  | Hazard Value 2 |
| --- | --- | --- |
| Limited | Little or no natural vegetative fuels are present. | 0 |
| Grass | Very little shrub or timber is present, generally less than one-third of the area. Main fuel is generally less than two feet in height. Fires are surface fires that move rapidly through cured grass and associated material. (Fuel model 1) | 3 |
| Grass | Open shrub lands and pine stands or scrub oak stands that cover one-third to two-thirds of the area. Main fuel is generally less that two feet in height. Fires are surface fires that spread primarily through the fine herbaceous fuels, either curing or dead. (Fuel model 2) | 3 |
| Grass | Beach grasses, prairie grasses, marshland grasses and wild or cultivated grains that have not been harvested. Main fuel is generally less than four feet in height, but considerable variation may occur. Fires are the most intense of the grass group and display high rates of spread under the influence of wind.(Fuel model 3) | 3 |
| Shrubs | Stands of mature shrubs have foliage known for its flammability, such as gorse, manzanita and snowberry. Main fuel is generally six feet or more tall. Fires burn with high intensity and spread very rapidly. (Fuel model 4) | 3 |
| Shrubs | Young shrubs with little dead material and having foliage not known for its flammability, such as laurel, vine maple and alders. Main fuel is generally three feet tall or less. Fires are generally carried in the surface fuels and are generally not very intense. (Fuel model 5) | 1 |
| Shrubs | Older shrubs with foliage having a flammability less than fuel model 4, but more than fuel model 5. Widely spaced juniper and sagebrush are represented by this group. Main fuel is generally less than six feet in height. Fires will drop to the ground at low wind speeds and in stand openings. (Fuel model 6) | 2 |
| Timber | Areas of timber with little undergrowth and small amounts of litter buildup. Healthy stands of lodgepole pine, spruce, fir and larch are represented by this group. Fires will burn only under severe weather conditions involving high temperatures, low humidity and high winds. (Fuel model 8) | 1 |
| Timber | Areas of timber with more surface litter than fuel model 8. Closed stands of healthy ponderosa pine and white oak are in this fuel model. Spread of fires will be aided by rolling or blowing leaves. (Fuel model 9) | 2 |
| Timber | Areas of timber with heavy buildups of ground litter caused by over-maturity or natural events of wind throw or insect infestations. Fires are difficult to control due to large extent of ground fuel. (Fuel model 10) | 3 |

1. Some areas may contain vegetative fuels other than those listed in Table 2.  Additional natural fuel hazard factors may be found  in “Aids to Determining Fuel Models for Estimating Fire Behavior” published by the Forest Service, USDA Intermountain Forest and Ranger Experiment Station in 1982 as General Technical Report INT-122.  Vegetative fuel hazard factors determined using General Technical Report INT-122 shall be used  as alternative factors, for review under this chapter, as the facts warrant.

2. Due to various factors, such as variations in local vegetation species or vegetation conditions, the fuel models used in Table 2 may not accurately portray wildfire behavior.  The Chief may make modifications to the hazard values as necessary to accurately reflect the following characteristics:

(a) A hazard value of 1 shall describe vegetation that typically produces a flame length of up to 5 feet, a wildfire which exhibits very little spotting, torching, or crowning, and which results in a burned area that can normally be entered within 15 minutes.

(b) A hazard value of 2 shall describe vegetation that typically produces a flame length of 5 to 8 feet, a wildfire which exhibits sporadic spotting, torching, or crowning, and which results in a burned area that can normally be entered within one hour.

(c) A hazard value of 3 shall describe vegetation that typically produces a flame length of over 8 feet, a wildfire that exhibits frequent spotting, torching, or crowning, and which results in a burned area that normally cannot be entered for over one hour

- Natural Vegetative Fuel Distribution Hazard Factor Value .  To determine the natural vegetative fuel distribution hazard factor value:

- Determine the percentage of each individual area that is covered by vegetation.

- Using the calculated percentage, assign a value using Table 3.

TABLE 3

NATURAL VEGETATIVE FUEL DISTRIBUTION

HAZARD FACTOR

| Natural Vegetative Fuel Distribution | Hazard Value |
| --- | --- |
| 0 to 10% of the area | 0 |
| 10 to 25% of the area | 1 |
| 25 to 40% of the area | 2 |
| 40 to 100% of the area | 3 |

24.51.040 Map Revision Process

A.	Wildfire Hazard Zones may be applied to or removed from areas of the City as follows:

1.	During periodic review by the Chief, based upon the criteria listed in section 24.51.030.  Periodic review shall occur every 5 years.

2.	Upon request to the Chief by any property owner, prior to periodic review, on the grounds that conditions have changed.

B.	Prior to applying the Wildfire Hazard Zone to any property the Chief shall provide notice of such proposed zoning and provide a date for a public hearing.

The notice shall be sent to all properties to which the zone would be applied.  The notice shall be sent fourteen days prior to the date of the hearing.  Extensions of time for the hearing may be requested and may be provided by the Chief.  The notice shall provide information regarding the City’s intention to apply the Wildfire Zone, the reasons therefore and the time and place for the hearing.

Within 7 days of the hearing the Chief shall issue a written decision, based upon the criteria listed above, and which shall include findings supporting that decision and shall contain information regarding the right to appeal the Chief’s decision to the Bureau of Development Service’s Appeals Board (Board).  A copy of the decision shall be sent to all properties that received notice of the City’s intention to include these properties within a Wildfire Hazard Zone.

C.	When a property owner provides the Chief with a written request that the Wildfire Hazard Zone be removed from specific property the Chief shall consider the request and, based upon the criteria listed above, shall either approve or deny the request.

Such action by the Chief shall occur within 14 days of the date of the request and shall be in writing, shall include findings based upon the facts and criteria and shall contain information regarding the right to appeal the Chief’s decision to the Board.  This decision shall be mailed to the property owner requesting the change in status.

24.51.050 Appeals of Decisions Made by the Chief

Notwithstanding any contradictory portion of Code section 24.10.080:

A.	Any decision made by the Chief, regarding the application of a Wildfire Hazard Zone to any area in the City, may be appealed to the Bureau of Development Services Board of Appeals (Board) solely in accordance with this subsection.  In considering such appeals the Board shall act solely in accord with this section.

B.	Such appeal shall be in writing and shall be filed with the Board within fourteen days of the date of the Chief’s decision.  The appeal shall include a statement regarding the elements of the Chief’s decision with which the appellant takes issue.  Reference to facts and the criteria listed above, is required.

C.	A copy of the appeal shall be provided to the Chief at the same time that it is filed with the Board.  The Chief shall have fourteen days from the date of the appeal to respond, in writing, to the Board and all appellants.

D.	The Board shall issue a notice of a hearing date and the place and time of the hearing.  Notice shall be provided to the appellants and the Chief.

E.	The Board shall then hold a hearing upon any such appeal.  After considering the issues raised on appeal, and the reasonableness of the Chief’s interpretation of applicable criteria, the Board shall, by majority vote, affirm or modify the Chief’s decision.  The Board’s decision shall be based solely upon the criteria set out in this Chapter and shall include findings addressing the facts and the criteria. The decision of the Board shall have full force and effect.  A certified copy of the decision shall be delivered to the appellant.

Any appeal of the Board’s decision shall by writ of review.

24.51.060 General

A.	In addition to the other City codes, all structures located in wildfire hazard zones as identified in the Wildfire Hazard Zone map shall meet the applicable requirements in the State of Oregon Structural Specialty Code or the One and Two Family Dwelling Specialty Code as applicable.

B.	The requirements in Chapter 24.75, Uniform Building Address System, supercede the requirements found in OSSC Appendix Chapter 5, Section 510.4.3, for premises identification.

Passed by the Council,  April 30, 2003					GARY BLACKMER

Auditor of the City of Portland

By  /S/ Susan Parsons

Commissioner Erik Sten																					Deputy

Carol Ann Boucher

April 15, 2003

BACKING SHEET INFORMATION

AGENDA NO.   378 400-2003

ACTION TAKEN:  APRIL 23, 2003 PASSED TO SECOND READING APRIL 30, 2003 9:30AM

ORDINANCE/RESOLUTION/COUNCIL DOCUMENT NO.   177433

| COMMISSIONERS VOTED AS FOLLOWS: |  |  |
| --- | --- | --- |
|  | YEAS | NAYS |
| FRANCESCONI | X |  |
| LEONARD | X |  |
| SALTZMAN | X |  |
| STEN | X |  |
| KATZ | X |  |