# SHEARWALL SCHEDULE

EXTERIOR SHEATHING AT ADDITION TO BE 15/32” OSB

EXCEPT AS NOTED, PROVIDE:

10d  COMMON @ 6” ALL PANEL EDGES, BLOCKED

10d  COMMON @ 12” FIELD

10d  COMMON @ 6” STUD SOLE PLATE

10d  COMMON @ 5” RIM TOENAIL TO TOP PLATE BELOW

10d  COMMON @ 4” ALL PANEL EDGES, BLOCKED

10d  COMMON @ 12” FIELD

10d  COMMON @ 4” STUD SOLE PLATE

10d  COMMON @ 3” RIM, TOENAIL TO TOP PLATE BELOW

15/32” OSB EACH FACE OF STUDS

10d  COMMON @ 2” ALL PANEL EDGES, BLOCKED

10d  COMMON @ 12” FIELD

2 ROWS 16d COMMON @ 3” AT SOLE PLATE

PROVIDE 4X6 FRAMING AT ALL PANEL EDGES

SUPPLEMENT EXISTING NAILING AS REQUIRED TO ACHIEVE REQUIRED NAILING.

8d COMMON @ 12”  IS ACCEPTABLE AT EXISTING FIELD NAILING.

HOLDOWN SCHEDULE

SIMPSON CMST 14 LAID VERTICALLY OVER SHEATHING WITH 2 ROWS OF (10)16d

@ 3-1/2” TO DBL STUDS ABOVE AND BELOW FLOOR (40 NAILS TOTAL)

SIMPSON CS16 STRAP LAID VERTICALLY OVER SHEATHING WITH (10)16d @ 4” O/C

TO STUD ABOVE & BELOW FLOOR  (20 NAILS TOTAL)

SIMPSON HD 14A ABOVE & BELOW FLOOR.  (4) 1” DIAMETER MB

AT (4) 2X STUDS EACH HOLDOWN.  1” DIAMETER THREADED CONNECTOR ROD.

SIMPSON HD8 ABOVE & BELOW FLOOR:  (3)  7/8” DIAMETER MB AT

4X STUD EACH HOLDOWN.  7/8” DIA. THREADED CONNECTOR ROD.

SIMPSON  HD2.  (10) ¼” DIA. X 3” SDS SCREWS AT DOUBLE STUDS.  5/8” DIA.

THREAD ROD IN ¾” DIA. X 6” HOLE FILLED WITH SIMPSON SET EPOXY.  SPECIAL INSPECTION.  SSTB 16 AT NEW CONCRETE.

SIMPSON HD14A EACH FACE OF 6X6 POST.  (4) 1” DIA. MB @ POST.  1” DIA. X  36”

THREAD ROD ANCHOR EACH HD							6”

SIMPSON HD8A EACH FACE OF 6X6 POST.  (3) 7/8  DIA. MB @ POST

7/8” DIA. THREADED ROD IN 1” DIA. X 15” HOLE FILLED WITH SIMPSON SET EPOXY.

SPECIAL INSPECTION.